|  | Spartan Chemical Company, Inc. Material Safety Data Sheet |
| --- | --- |

SECTION I: PRODUCT INFORMATION

SECTION I: PRODUCT INFORMATION

| Product Name or Number (as it appears on label): | Product Division: |
| --- | --- |
| FOAMING ACID CLEANER FP Product Number: 3081 | Janitorial |

| Spartan Chemical Company, Inc. | Product/Technical Information: | 1-(800)-537-8990 |
| --- | --- | --- |
| 1110 Spartan Drive | Medical Emergency: | 1-(888)-314-6171 (24 hours) |
| Maumee OH 43537 | Chemical Leak/Spil Emergency: | CHEMTREC 1-(800) 424-9300 (24 hours) |

| Shipping Description: | Corrosive liquids, n.o.s., 8, UN 1760, III (contains phosphoric acid) |
| --- | --- |

| NFPA Ratings: |  | HMIS Ratings: |  |
| --- | --- | --- | --- |
| Health: | 3 - Serious | Health: | 3 - Serious |
| Fire: | 0 - Minimal | Fire: | 0 - Minimal |
| Reactivity: | 0 - Minimal | Reactivity: | 0 - Minimal |
|  |  | Pers. Prot. Equip.: | See Section VIII |

SECTION II: HAZARDOUS INGREDIENTS

SECTION II: HAZARDOUS INGREDIENTS
(Listed when present at 1% or greater, carcinogens at 0.1% or greater) All component chemicals are listed or exempted from listing on the "TSCA Inventory" of chemical substances maintained by the U.S. Environmental Protection Agency.

| Chemical Name(s) | % W | CAS Registry No. | Table Z- 1 - A |  |  | NTP, IARC or OSHA Carcinogen |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TWA mg/m3 | STEL mg/m 3 | CELING mg/m3 |  |
| Phosphoric acid | 30-40 | 7664-38-2 | 1 | 3 (NIOSH) | Not Established | No |
| Alcohol ethoxylate | 1-5 | 68439-46-3 | Not Established | Not Established | Not Established | No |

SECTION III: PHYSICAL DATA

SECTION III: PHYSICAL DATA

| Boiling Point: | > 212 °F | Vapor Pressure: | Unknown |
| --- | --- | --- | --- |
| Vapor Density (AIR = 1): | Unknown | Solubility in Water: | Complete |
| pH: | < 1.0 | Specific Gravity (H2O=1): | 1.21 |
| Evaporation Rate (but.ace.=1): | <1 | Percent Solid by Weight: | 30-35 |
| Physical State: | Liquid |  |  |

| Appearance & Odor: | Pale red liquid with mild odor |
| --- | --- |

SECTION IV: FIRE & EXPLOSIVE HAZARD DATA

SECTION IV: FIRE & EXPLOSIVE HAZARD DATA

| Flash Point: | None to 212°F | Method Used: | ASTM-D56 |
| --- | --- | --- | --- |
| Flammable Limits: | Not Established | Flame Extension: | N/A |

| Extinguishing Media: | Product does not support combustion. Use extinguishing media appropriate for surrounding fire. |
| --- | --- |
| Special Fire Fighting Procedures: | Wear NIOSH approved self-contained breathing apparatus and protective clothing. Cool fire-exposed containers with water spray. |
| Unusual Fire & Explosive Hazards: | Evolves hydrogen on contact with most metals. Combustion products are toxic. |

SECTION V: HEALTH HAZARD DATA

SECTION V: HEALTH HAZARD DATA

| Threshold Limit Value: | Not Established | Primary Routes of Entry: | Inhalation, Skin Contact, Eyes, Ingestion |
| --- | --- | --- | --- |

| Effects of Overexposure- Conditions to Avoid: | CORROSIVE. CAUSES IRREVERSIBLE EYE DAMAGE AND SKIN BURNS: Symptoms include chemical burns, pain, redness, swelling of the conjunctiva, tissue destruction and possible permanent corneal damage. HARMFUL IF SWALLOWED. Causes damage or chemical burns to esophagus and mucous membranes with symptoms of pain, nausea, vomiting and diarrhea. Aspiration of product into the lungs may occur during ingestion or vomiting, resulting in lung injury. HARMFUL IF INHALED. Breathing spray mist may cause coughing, difficulty breathing and possibly tissue injury. Do not get in eyes, skin or clothing. Do not taste or swallow. Avoid breathing spray mist. Use with adequate ventilation. Wash thoroughly with soap and water after handling. |
| --- | --- |
| Conditions Aggravated by Use: | Use of this product may aggravate preexisting skin; eye and respiratory disorders including asthma and dermatitis. |

Emergency & First Aid Procedures:

| Eyes: | Immediately flush eyes with plenty of water for at least 15 minutes. Remove contact lenses. Get medical attention. |
| --- | --- |
| Skin: | Remove contaminated clothing and shoes. Flush skin with water for at least 15 minutes . Get medical attention. Wash contaminated clothing before reuse. Destroy contaminated shoes. |
| Ingestion: | Call a poison control center or doctor immediately for treatment advice. Do not induce vomiting. Drink one or two glasses of water to dilute product. Do not give anything by mouth to an unconscious person. |
| Inhalation: | In case of respiratory irritation move person to fresh air. If not breathing; give artificial respiration. If breathing is difficult; give oxygen. Get immediate medical attention. |

Note to Physician: Probable mucosal damage may contraindicate the use of gastric lavage.

SECTION VI: REACTIVITY DATA

SECTION VI: REACTIVITY DATA

| Stability: | Stable | Incompatible Materials: | Concentrated alkali; hypochlorites (chlorine bleach); Aluminum and other reactive metals. |
| --- | --- | --- | --- |
| Hazardous Decomposition Products: | Oxides of phosphorus | Hazardous Polymerization: | Will Not Occur |

SECTION VII: SPILL OR LEAK PROCEDURES

SECTION VII: SPILL OR LEAK PROCEDURES

| Steps to be Taken in Case Material is Released or Spilled: | Contain large spills and transfer the material to appropriate containers for reclamation or disposal. Absorb remaining material or small spills with an inert material and dispose in accordance with Federal, State and Local regulations. |
| --- | --- |
| Waste Disposal Method: | Follow Federal, State and Local regulations. |

SECTION VIII: SPECIAL PROTECTION INFORMATION

SECTION VIII: SPECIAL PROTECTION INFORMATION

| Respiratory Protection: | Not expected to be required if good general ventilation is provided. However if exposure limits are exceeded (see Section II) or if respiratory irritation occurs, use of an appropriate NIOSH approved respirator with a full facemask should be considered. |
| --- | --- |
| Ventilation: | Provide good general ventilation. Local exhaust ventilation may be necessary for some operations. |
| Protective Gloves(Specify Type): | Wear rubber or other impervious gloves. |
| Eye Protection(Specify Type): | Wear splash goggles to prevent contact with product. |
| Other Protective Equipment: | Use of impervious apron, face shield and other protective equipment should be considered in order to prevent or minimize contact with this product. Eye wash stations and washing facilities should be readily accessible. See 29 CFR 1910.132-138 for further guidance. |

SECTION IX: SPECIAL PRECAUTIONS

SECTION IX: SPECIAL PRECAUTIONS

| Precautions; Handling & Storing: | Keep from freezing. |
| --- | --- |
| Other Precautions: | Keep out of reach of children |

| © SCC 06/01/2007 | Name: | Ronald T. Cook | Title: | Manager, Regulatory Affairs |
| --- | --- | --- | --- | --- |
| FOAMING ACID CLEANER FP | Effective Date: | 06/01/2007 | Supercedes: | 03/15/2005 |
| Ref: 29 CFR 1910.1200 (OSHA) | Changes: | Revised formula |  |  |

This document has been prepared using data from sources considered technically reliable. It does not constitute a warranty, express or implied, as to the accuracy of the information contained herein. Actual conditions of use and handling are beyond sellers control. User is responsible to evaluate all available information when using product for any particular use and to comply with all Federal, State and Local laws and regulations.